Got it—here’s a focused, code-level comparison of the computational procedures the two implementations perform. I’ll call them UMAT (ABAQUS) vs numgeo module.

# What’s the same (procedural core)

- Subincrementation: both split the total strain increment into nsub substeps based on a tolerance and use sub_dstrain = -dε / nsub in geotechnical sign for tensors, mechanical for Voigt.

- Bounding-surface projection: at the start of each substep (and again after the elastic trial) both project the stress to a bounding surface, then recompute invariants.

- Elastic trial: compute Sanisand elastic stiffness Gh,Kh,λG_h, K_h, \lambda, form CC, and do σtrial=σ−C:Δε\sigma^{\text{trial}} = \sigma - C : \Delta\varepsilon.

- Yield check & plastic correction: identical structure—compute b0,n0,h,ψ,αD,αB,Kp,Ad,Dil,Gh,Khb_0, n_0, h, \psi, \alpha_D, \alpha_B, K_p, A_d, \mathrm{Dil}, G_h, K_h, then plastic multiplier

Lplast=2Gh(n0:dev(Δε))−(n0:s/p) tr(Δε)Kp+2Gh(B−C 3det⁡(n0))−Kh Dil (n0:s/p)L_{\text{plast}}=\frac{2G_h (n_0:\mathrm{dev}(\Delta\varepsilon))-(n_0:s/p)\,\mathrm{tr}(\Delta\varepsilon)}{K_p+2G_h\left(B-C\,3\det(n_0)\right)-K_h\,\mathrm{Dil}\,(n_0:s/p)}

and iterate adding a plastic stress increment until the yield function is satisfied; fallback mapping to the yield surface if needed.

- State updates: same formulas for back-stress α\alpha, plastic strain counter, and fabric zz (dilatancy-driven).

- Consistent tangent actually returned: both effectively return elastic CC accumulated over substeps; the additional plastic stiffness contributions are present in comments but not assembled.

# Key procedural differences

- Stress projection control & angle

- UMAT: always calls proj_bounding every substep with a fixed angle factor ((alphacc*3/(6+alphacc))*2.0) and enforces a minimum mean stress via pmin.

- numgeo: projection is conditional (only if material_options%use_projection_* flags are set) and the angle is additionally scaled by a runtime proj_angle_factor.
⇒ Practically: numgeo may skip projection (depending on options), and even if enabled, the projection “tightness” can differ from UMAT.

- Phantom/artificial viscosity

- UMAT: enables phantom viscosity by default (use_visco=.true.), subtracts a viscous stress σvis/Δt\sigma_\text{vis}/\Delta t after the mechanical update, and stores the viscous part in state variables.

- numgeo: viscosity hooks exist but are commented out; behavior is governed by material_options%use_visco, and in the provided code path viscosity is not applied.
⇒ Procedurally, UMAT performs an extra constitutive evaluation (phantomvisco) and superposes it; numgeo doesn’t.

- Elastic modulus evaluation (pressure clipping)

- UMAT: clamps the pressure used for modulus in elastic() to pc∈[1, 3000] (units of pat) before computing Gh,KhG_h, K_h.

- numgeo: no pressure clipping; uses the current pp directly.
⇒ Different CC in small/large pp regimes; affects both the trial stress and the (elastic) tangent returned.

- Substepping controls

- UMAT: tol = 2e-5, maxnint = 5000, nsub_plastic_max = 20.

- numgeo: tighter tol = 1e-6, maxnint = 50000, nsub_plastic_max = 50.
⇒ numgeo tends to take more (smaller) substeps and allows more plastic sub-iterations before falling back.

- Void-ratio bounds during update

- UMAT: clamps ee to [0.50 ec0, 0.99 ec0][0.50\,e_{c0},\,0.99\,e_{c0}].

- numgeo: clamps ee to [0.30 ec0, 1.00 ec0][0.30\,e_{c0},\,1.00\,e_{c0}].
⇒ numgeo allows looser evolution of ee, especially towards denser and looser ends. This feeds back into Gh,KhG_h, K_h.

- Initialization & mapping details

- UMAT: initializes α\alpha early if ∥α∥<1e−1\|\alpha\| < 1e-1; on fallback mapping it explicitly copies α0←α\alpha_0 \leftarrow \alpha.

- numgeo: initialization tied to first increment flags; on fallback mapping it does not always reset α0\alpha_0 in that spot (though it updates α0\alpha_0 when (α−α0):n0<0(\alpha-\alpha_0):n_0<0 earlier).
⇒ Small path-dependent differences after non-convergence or mapping.

- Returned tangent assembly

- UMAT: accumulates Eelastic/nsub unconditionally; plastic contribution not added; viscosity tangent not added.

- numgeo: same idea (Eelastic/nsub), with a commented-out plastic part (dds_dde_plastic) and optional determinant “regularization” also commented.
⇒ Both return essentially elastic stiffness, but numgeo contains scaffolding for a fuller consistent tangent.

- Configurability / options surface

- UMAT: self-contained, fixed choices (projection always on, viscosity always on, fixed angles/bounds).

- numgeo: many behaviors are option-gated (projection, viscosity) via material_options, exposing more runtime control for the same algorithmic steps.

# Bottom line

Algorithmically they implement the same constitutive procedure, but you’ll see different numerical behavior because the numgeo version:

- can skip or scale stress projection via options,

- omits the viscosity superposition in the shown code path,

- uses tighter substepping and wider ee-bounds, and

- does no pressure clipping for the elastic moduli.

If you want them to produce closer trajectories, align these knobs: enable the same projection (and angle factor), adopt the same pressure clipping for CC, harmonize tol/maxnint/nsub_plastic_max, match the ee-clamps, and either enable artificial viscosity in numgeo or disable it in UMAT.

If you’d like, I can draft a short “equivalence checklist” (side-by-side settings/code edits) to synchronize them.